Clarification of Internet and Repeater Gateway Linking Operating Procedures.

The Notice of Variation (NoV) issued for Internet Linking allows the Linking of Amateur Radio to the Internet.  A Repeater Gateway NoV allows the Linking of Amateur Radio to the Internet via a Repeater.

This facility should be used on an attended basis only.  There is no provision at present for unattended use of the Internet Linking facility.

For further information on this service please contact the Amateur Radio Section on 020 7211 0160.

Attached are examples of the NoV's issued for the above.

# WIRELESS TELEGRAPHY ACT 1949

NOTICE OF VARIATION OF AMATEUR RADIO LICENCE

Name of Licensee:			 	(“the Licensee”)

Licensee’s Main Station Address:

(“the Main Station Address”):

Licensee’s Call Sign:

Date of Issue of the  				 (“the Date of Issue”)

Notice of Variation:

The Secretary of State, in exercise of the power conferred by section 1(4) of the Wireless Telegraphy Act 1949, hereby varies the Amateur Radio Licence (“the Licence”) as follows:

1.1	Terms and expressions defined in the Licence shall have the same meaning herein except where the context requires otherwise.

1.2	In this Notice “the Booklet” means the Amateur Radio Licence (A) or (B) Terms, Provisions and Limitations Booklet BR68/N (June 1997 Iss 1 Rev 5 edition) as amended from time to time the terms of which form part of the Licence.

2	As and from the Date of Issue the Licence shall be read and construed as if:-

(1)	the following were added to the end of Note (h) to the Booklet:-

ESTABLISHMENT OF GATEWAY

Subject to paragraphs (i) to (v) below, the Licensee shall be permitted to connect the Station to non-Amateur networks for the purposes of establishing a gateway (“Gateway”) at the Main Station Address.

LIMITATIONS ON OPERATION

(i)	The Licensee shall operate the Gateway only on an attended basis at or with:

(a)	the call sign specified in the Gateway Schedule;

(b)	the frequencies specified in the Gateway Schedule;

(c)	a power not exceeding that specified in the Gateway Schedule;

(d)	the antenna characteristics specified in the Gateway Schedule; and

(e)	the mode(s) of emission specified in the Gateway Schedule.

CONTROL OF THE GATEWAY

(iii)	(a)	The Licensee shall not conduct operations on the Gateway or operate the Gateway unless he has given at least 7 days written notice to the Manager of the Local Customer Service office in whose  region the Gateway is located.

(b)	The Licensee shall be responsible for monitoring all traffic entering the Gateway to ensure that the content of Messages to be transmitted are in accordance with the terms of the Amateur Radio Licence (“the Terms”) and the Wireless Telegraphy(Content of Transmission) Regulations 1988 (“the 1988 Regulations”) and for closing the Gateway down immediately where such Messages are found to be in contravention of the Terms or the 1988 Regulations. The Licensee is not required to monitor traffic leaving the Gateway but if the Licensee becomes aware of Messages leaving the Gateway which are found to be in contravention of the Terms or the 1988 Regulations, the Licensee must close the Gateway down immediately.

IDENTIFICATION REQUIRMENTS

(iv)	The Licensee shall transmit the Gateway call sign:

(a)	at the beginning of each transmission and when the period of transmission is longer than 15 minutes, at the end of each interval of 15 minutes; and

(b)	by Morse telegraphy using an audio tone of 500-1500 Hz at a peak deviation of 500-1000 Hz superimposed on the link audio at a  speed not exceeding  20 words per minute.

GATEWAY LOG

(v)	The Licensee shall keep a separate Log for the Gateway showing the dates and times (in Co-ordinated Universal Time (UTC)) during which the Gateway is switched on and available for use, or during which test transmissions were made; and

(2)	the Gateway Schedule to the Notice of Variation attached hereto as amended or replaced from time to time by notice from the Secretary of State were inserted after the Notes to the Schedule to the Booklet

On behalf of the Secretary of State for Trade and Industry

31 July 2026

SCHEDULE (1) TO THE NOTICE OF VARIATION

Issued to:

Call sign:

Date of issue:

Gateway

Gateway call sign:

Date of issue of NoV:

Frequencies

- Transmitter:

- Receiver:

Mode(s) of emission:

Maximum Transmitter Power:	 (erp)

Antenna Characteristics

- Antenna type:

- Direction of maximum radiation (degrees East of True North):	N/A

- Polarisation:	Vertical

- Height above ground:	MASL

Any change to the Gateway Station which would deviate from the above will need the prior written approval of the Secretary of State.

SCHEDULE  (2) TO THE NOTICE OF VARIATION

Issued to:

Call sign:

Date of issue:

Gateway

Gateway call sign:

Date of issue of NoV:

Frequencies

- Transmitter:

- Receiver:

Mode(s) of emission:

Maximum Transmitter Power:		 dbw (erp)

Maximum Deviation			+/-2.5 kHz

Antenna Characteristics

- Antenna type:

- Direction of maximum radiation (degrees East of True North):	N/A

- Polarisation:	Vertical

- Height above ground:	MASL

Any change to the Gateway Station which would deviate from the above will need the prior written approval of the Secretary of State.

# WIRELESS TELEGRAPHY ACT 1949

NOTICE OF VARIATION OF AMATEUR RADIO LICENCE (repeater gateway)aw

Name of Licensee:				 (“the Licensee”)

Licensee’s Main Station Address:

(“the Main Station

Address”):

Licensee’s Call Sign:

Date of Issue of the  				(“the Date of Issue”)

Notice of Variation:

The Secretary of State, in exercise of the power conferred by section 1(4) of the Wireless Telegraphy Act 1949, hereby varies the Amateur Radio Licence (“the Licence”) as follows:

1.1	Terms and expressions defined in the Licence shall have the same 	meaning herein except where the context requires otherwise.

1.2	In this Notice “the Booklet” means the Amateur Radio Licence (A) or (B) Terms, Provisions and Limitations Booklet BR68 (June 1997 Iss 1 Rev 5 edition) as amended from time to time the terms of which form part of the Licence.

2	As and from the Date of Issue the Licence shall be read and construed as if:-

(1)	the following were added to the end of Note (h) to the Booklet:-

ESTABLISHMENT OF GATEWAY

Subject to paragraphs (i) to (v) below, the Licensee shall be permitted to connect the Station to non-Amateur networks for the purposes of establishing a gateway (“Gateway”) at the Main Station Address.

LIMITATIONS ON OPERATION

(i)	The Licensee shall operate the Gateway only on an attended basis at or with:

(a)	the call sign specified in the Gateway Schedule;

(b)	the frequencies specified in the Gateway Schedule;

(c)	a power not exceeding that specified in the Gateway Schedule;

(d)	the antenna characteristics specified in the Gateway Schedule; and

(e)	the mode(s) of emission specified in the Gateway Schedule.

(ii)	The Licensee shall only connect the Gateway with the Repeater identified in the Schedule.

CONTROL OF THE GATEWAY

(iii)	(a)	The Licensee shall not conduct operations on the Gateway or operate the Gateway unless he has given at least 7 days written notice to the Manager of the Local Customer Service office in whose  region the Gateway is located.

- The Licensee shall be responsible for monitoring all traffic entering the Gateway to ensure that the content of Messages to be transmitted are in accordance with the terms of the Amateur Radio Licence (“the Terms”) and the Wireless Telegraphy (Content of Transmission) Regulations 1988 (“the 1988 Regulations”) and for closing the Gateway down immediately where such Messages are found to be in contravention of the Terms or the 1988 Regulations. The Licensee is not required to monitor traffic leaving the Gateway but if the Licensee becomes aware of Messages leaving the Gateway which are found to be in contravention of the Terms or the 1988 Regulations, the Licensee must close the Gateway down immediately.

- The Licensee shall inform the Keeper of the Repeater linked to the Gateway whenever the Gateway is or is to be switched off for periods exceeding 72 hours in total.

IDENTIFICATION REQUIRMENTS

(iv)	The Licensee shall transmit the Gateway call sign:

(a)	at the beginning of each transmission and when the period of transmission is longer than 15 minutes, at the end of each interval of 15 minutes; and

(b)	by Morse telegraphy using an audio tone of 500-1500 Hz at a peak deviation of 500-1000 Hz superimposed on the link audio at a  speed not exceeding  20 words per minute.

GATEWAY LOG

(v)	The Licensee shall keep a separate Log for the Gateway showing the dates and times (in Co-ordinated Universal Time (UTC)) during which the Gateway is switched on and available for use, or during which test transmissions were made; and

(2)	the Gateway Schedule to the Notice of Variation attached hereto as amended or replaced from time to time by notice from the Secretary of State were inserted after the Notes to the Schedule to the Booklet.

On behalf of the Secretary of State for Trade and Industry

31 July 2026

SCHEDULE TO THE NOTICE OF VARIATION

Issued to:

Call sign:

Date of issue:

Gateway

Gateway call sign:

Repeater call sign:

Date of issue of NoV:

Frequencies

- Transmitter:

- Receiver:

Mode(s) of emission:

Maximum Transmitter Power:		dBW (erp)

Antenna Characteristics

- Antenna type:

- Direction of maximum radiation (degrees East of True North):	N/A

- Polarisation:	Vertical

- Height above ground:	M

Any change to the Gateway Station which would deviate from the above will need the prior written approval of the Secretary of State.