# Radiocommunications Agency consultation document: Proposed Changes to Amateur Radio licences

## 1.	Introduction

This consultation document invites comments on proposed changes to Amateur Radio licences, concerning:

- the requirement to take a 5 wpm Morse test for access to bands below 30 MHz (see Section 2);

- allocations above 75 GHz (see Section 3); and

- Intermediate licensees’ access to amateur satellites (see Section 3).

### Why is the Agency consulting on these changes?

The Communications Bill, currently going through Parliament, will oblige the Agency to consult on proposed changes to WT Act licences, including amateur licences. We intend to follow the spirit of that requirement pending the completion of the Bill’s passage.

## 2.	Amateur operation below 30 MHz – requirement for Morse code test at 5 words per minute

### Introduction and background

At present, radio amateurs wishing to operate in amateur bands below 30 MHz must pass a test demonstrating their Morse code abilities. Article 25.5 of the international Radio Regulations obliges all national administrations (including the Agency) to conduct such tests for access to these bands.

The future need for this obligation is on the agenda of this year’s World Radio Conference (WRC-03), being held in Geneva from 9 June to 4 July. Having consulted the amateur community, the UK believes that the Morse test requirement should be removed. We consider it likely that this will be the majority view at the conference.

To speed up the process of implementing WRC-03’s decisions, the Agency is now consulting on a proposal to remove the Morse requirement for access to amateur bands below 30 MHz. Please note that, if WRC-03 does not remove the requirement for Morse code tests, the Agency cannot proceed with the proposed removal within the UK. We are consulting now only so that the overall process may be accomplished more quickly.

We propose to merge the A and B licence classes at Full and Intermediate levels, so that the UK licences comprise Foundation, Intermediate and Full. All licensees at each level will have access to all the frequencies forming part of their respective licences.

Each licensee will keep his/her current call sign. If a licensee holds both an A and a B licence at the same level (which, if these proposals are accepted, will offer identical privileges), s/he will be asked to choose which call sign to retain by not renewing the other licence when it is next due for renewal. (The discarded licence may or may not be his/her first licence to fall for renewal.)

### Proposed changes to BR68

Title		change to “Amateur Radio (Full) Licence Terms, Provisions and Limitations Booklet BR68”

1st paragraph	change to “These terms, provisions and limitations shall be read as an integral part of the Amateur Radio (Full) Licence.”

Clause 2(2)	delete in entirety.

Clause 2(10)(a)	change to “authorise any club member who holds an Amateur Radio (Full) Licence to use and supervise the operation of the station on his behalf under this Licence;”

Clause 3(1)(a)	delete “and” following the semicolon at the end of the clause and replace with “or”

Clause 3(1)(b)	change to “a Full Amateur Radio Examination Pass Certificate issued by or on behalf of the Secretary of State; or”

Clause 10(4)	delete in entirety.

Schedule	delete “Those licensed under an Amateur Radio Licence (B) may not transmit on those bands between 0.1357 and 29.700 MHz.”

Notes to the Booklet:

Note (f)	delete in entirety.

### Proposed changes to BR68/I

Title	change to “Amateur Radio (Intermediate) Licence Terms, Provisions and Limitations Booklet BR68/I”

1st paragraph	change to “These terms, provisions and limitations shall be read as an integral part of the Amateur Radio (Intermediate) Licence.”

Clause 2(2)	delete in entirety.

Clause 3(1)(a)	change to “an Amateur Radio (Novice) Examination Certificate issued by the City & Guilds of London Institute; or”

Clause 3(1)(b)	change to “a Amateur Radio (Intermediate) Examination Pass Certificate issued by or on behalf of the Secretary of State.”

Schedule	delete “Those licensed under an Amateur Radio (Intermediate) Licence (B) may not transmit on those bands between 0.1357 and 29.700 MHz.”

Notes to the Booklet:

Note (f)	delete in entirety.

## 3.	Amateur frequencies above 75 GHz/Satellite operation for Intermediate licensees

### Introduction and background

The World Radio Conference 1999 made changes to amateur allocations above 75 GHz. These changes have now come into effect, and we are seeking views on how to implement them.

With the introduction of the Foundation licence, the amateur community has suggested that Intermediate licensees should have access to amateur satellites. The Agency is pleased to adopt this suggestion, the implementation of which is also covered by this consultaion. We are also taking the opportunity to align the format of the Schedules to all three licences.

### Proposed changes to BR68

Change the licence schedule; a draft of the new schedule is attached as Annex 1.

### Proposed changes to BR68/I

Change the licence schedule; a draft of the new schedule is attached as Annex 2.

### Proposed changes to BR68/F

Change the licence schedule; a draft of the new schedule is attached as Annex 3. (This is simply a change of format to mirror the format of the Intermediate and Full licence schedules, by adding a column about the allocation to the Amateur Satellite Service. No actual changes of allocation are proposed; Foundation licensees do not have an allocation to the Amateur Satellite Service.)

## 4.	Responding to the consultation

Comments on any of the proposed changes should be made in writing or by email to:

Amateur Section

Radiocommunications Agency

Wyndham House

189 Marsh Wall

London

E14 9SX

Email: amateurcb@ra.gsi.gov.uk

Please ensure that we receive your comments by Friday 11 July.

Please note that this consultation concerns only the proposed changes. The Agency regrets that consultation responses cannot be taken by telephone, but questions on the consultation can be taken by the Agency’s Amateur Section on 020 7211 0160.

The Schedule to Terms, Provisions and Limitations Booklet BR68 
(Amateur Radio Licence)

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the | United Kingdom to: | Maximum Power level in Watts (dB relative to 1 W) PEP | Mode of Transmission |
|  | The Amateur Service | The Amateur Satellite Service |  |  |
| 0.1357-0.1378 | Secondary. Available on the basis of non-interference to other services (inside or outside the United Kingdom) | (not allocated) | 1 W (0 dBW) erp | Morse Telephony RTTY Data Facsimile SSTV |
| 1.810-1.830 | Primary. Available on the basis of non-interference to other services (outside the United Kingdom) |  | 400 W (26 dBW) |  |
| 1.830-1.850 | #### Primary |  |  |  |
| 1.850-1.950 | Available on the basis of non-interference to other services (inside or outside the United Kingdom) |  | 32 W (15 dBW) |  |
| 1.950-2.000 |  |  | 400 W (26 dBW) |  |
| 3.500-3.800 | Primary. Shared with other services |  |  |  |
| 7.000-7.100 | #### Primary | #### Primary |  |  |
| 10.100-10.150 | #### Secondary | (not allocated) |  |  |
| 14.000-14.250 | #### Primary | Primary |  |  |
| 14.250-14.350 |  | (not allocated) |  |  |
| 18.068-18.168 |  | Primary |  |  |
| 21.000-21.450 |  |  |  |  |
| 24.890-24.990 |  |  |  |  |
| 28.000-29.700 |  |  |  |  |
| 50.00-51.00 | Primary. Available on the basis of non-interference to other services outside the United Kingdom | (not allocated) |  |  |
| 51.00-52.00 | Secondary. Available on the basis of non-interference to other services inside or outside the United Kingdom |  | 100 W (20 dBW) |  |
| 70.00-70.50 |  |  | 160 W (22 dBW) |  |
| 144.0-146.0 | #### Primary | Primary | 400 W (26 dBW) |  |
| 430.0-431.0 | Secondary | (not allocated) | 40 W (16 dBW) erp | Morse Telephony RTTY Data Facsimile SSTV FSTV |
| 431.0-432.0 | Secondary. Not available for use within a 100 km radius of Charing Cross, London (51°30´30´´N, 00°07´24´´W) |  |  |  |
| 432.0- 435.0 | #### Secondary |  | 400 W (26 dBW) |  |
| 435.0-438.0 |  | Secondary |  |  |
| 438.0-440.0 |  | (not allocated) |  |  |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the | United Kingdom to | Maximum Power level in Watts (dB relative to 1 W) PEP | Mode of Transmission |
|  | The Amateur Service | The Amateur Satellite Service |  |  |
| 1240-1260 | Secondary | (not allocated) | 400 W (26 dBW) | Morse Telephony RTTY Data Facsimile SSTV FSTV |
| 1260-1270 |  | Secondary. Earth to space only |  |  |
| 1270-1325 |  | (not allocated) |  |  |
| 2310-2400 |  |  |  |  |
| 2400-2450 | Secondary. Users must accept interference from ISM users | Secondary. Users must accept interference from ISM users |  |  |
| 3400-3475 | Secondary | (not allocated) |  |  |
| 5650-5670 |  | Secondary. Earth to space only |  |  |
| 5670-5680 |  | (not allocated) |  |  |
| 5755-5765 | Secondary. Users must accept interference from ISM users |  |  |  |
| 5820-5830 |  |  |  |  |
| 5830-5850 |  | Secondary. Users must accept interference from ISM users. Space to Earth only |  |  |
| 10000-10125 | Secondary | (not allocated) |  |  |
| 10225-10450 |  |  |  |  |
| 10450-10475 |  | Secondary |  |  |
| 10475-10500 | (not allocated) |  |  |  |
| 24000-24050 | Primary. Users must accept interference from ISM users | Primary. Users must accept interference from ISM users |  |  |
| 24050-24150 | Secondary. May only be used with the written consent of the Secretary of State. Users must accept interference from ISM users | (not allocated) |  |  |
| 24150-24250 | Secondary |  |  |  |
| 47000-47200 | Primary | Primary |  |  |
| 75500-76000 | Primary (1) | Primary (1) |  |  |
| 76000-77500 | Secondary | Secondary |  |  |
| 77500-78000 | Primary | Primary |  |  |
| 78000-79000 | Secondary | (not allocated) |  |  |
| 79000-81000 |  | Secondary |  |  |
| 122250-123000 |  |  |  |  |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the United Kingdom to |  | Maximum Power level in Watts (dB relative to 1 W) PEP | Mode of Transmission |
|  | The Amateur Service | The Amateur Satellite Service |  |  |
| 134000-136000 | Primary | Primary | 400 W (26 dBW) | Morse Telephony RTTY Data Facsimile SSTV FSTV |
| 136000-141000 | Secondary | Secondary |  |  |
| 142000-144000 | Primary (1) | Primary (1) |  |  |
| 241000-248000 | Secondary | Secondary |  |  |
| 248000 -250000 | Primary | Primary |  |  |

ISM = Industrial, Scientific and Medical.

(1) Until 31 December 2006.

The Schedule to Terms, Provisions and Limitations Booklet BR68/I
(Amateur Radio (Intermediate) Licence)

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the | United Kingdom to: | Maximum Power level in Watts (dB relative to 1 W) PEP | Mode of Transmission |
|  | The Amateur Service | The Amateur Satellite Service |  |  |
| 0.1357-0.1378 | Secondary. Available on the basis of non-interference to other services (inside or outside the United Kingdom) | (not allocated) | 1 W (0 dBW) erp | Morse Telephony RTTY Data Facsimile SSTV |
| 1.810-1.830 | Primary. Available on the basis of non-interference to other services (outside the United Kingdom) |  | 50 W (17 dBW) |  |
| 1.830-1.850 | #### Primary |  |  |  |
| 1.850-1.950 | Available on the basis of non-interference to other services (inside or outside the United Kingdom) |  | 32 W (15 dBW) |  |
| 1.950-2.000 |  |  | 50 W (17 dBW) |  |
| 3.500-3.800 | Primary. Shared with other services |  |  |  |
| 7.000-7.100 | #### Primary | #### Primary |  |  |
| 10.100-10.150 | #### Secondary | (not allocated) |  |  |
| 14.000-14.250 | #### Primary | Primary |  |  |
| 14.250-14.350 |  | (not allocated) |  |  |
| 18.068-18.168 |  | Primary |  |  |
| 21.000-21.450 |  |  |  |  |
| 24.890-24.990 |  |  |  |  |
| 28.000-29.700 |  |  |  |  |
| 50.00-51.00 | Primary. Available on the basis of non-interference to other services outside the United Kingdom | (not allocated) |  |  |
| 51.00-52.00 | Secondary. Available on the basis of non-interference to other services inside or outside the United Kingdom |  |  |  |
| 70.00-70.50 |  |  |  |  |
| 144.0-146.0 | #### Primary | Primary |  |  |
| 430.0-431.0 | Secondary | (not allocated) | 40 W (16 dBW) erp | Morse Telephony RTTY Data Facsimile SSTV FSTV |
| 431.0-432.0 | Secondary. Not available for use within a 100 km radius of Charing Cross, London (51°30´30´´N, 00°07´24´´W) |  |  |  |
| 432.0- 435.0 | #### Secondary |  | 50 W (17 dBW) |  |
| 435.0-438.0 |  | Secondary |  |  |
| 438.0-440.0 |  | (not allocated) |  |  |

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the | United Kingdom to | Maximum Power level in Watts (dB relative to 1 W) PEP | Mode of Transmission |
|  | The Amateur Service | The Amateur Satellite Service |  |  |
| 1240-1260 | Secondary | (not allocated) | 50 W (17 dBW) | Morse Telephony RTTY Data Facsimile SSTV FSTV |
| 1260-1270 |  | Secondary. Earth to space only |  |  |
| 1270-1325 |  | (not allocated) |  |  |
| 2310-2400 |  |  |  |  |
| 2400-2450 | Secondary. Users must accept interference from ISM users | Secondary. Users must accept interference from ISM users |  |  |
| 3400-3475 | Secondary | (not allocated) |  |  |
| 5650-5670 |  | Secondary. Earth to space only |  |  |
| 5670-5680 |  | (not allocated) |  |  |
| 5755-5765 | Secondary. Users must accept interference from ISM users |  |  |  |
| 5820-5830 |  |  |  |  |
| 5830-5850 |  | Secondary. Users must accept interference from ISM users. Space to Earth only |  |  |
| 10000-10125 | Secondary | (not allocated) |  |  |
| 10225-10450 |  |  |  |  |
| 10450-10475 |  | Secondary |  |  |
| 10475-10500 | (not allocated) |  |  |  |
| 24000-24050 | Primary. Users must accept interference from ISM users | Primary. Users must accept interference from ISM users |  |  |
| 24050-24150 | Secondary. May only be used with the written consent of the Secretary of State. Users must accept interference from ISM users | (not allocated) |  |  |
| 24150-24250 | Secondary |  |  |  |
| 47000-47200 | Primary | Primary |  |  |
| 75500-76000 | Primary (1) | Primary (1) |  |  |
| 142000-144000 | Primary (1) | Primary |  |  |
| 248000-250000 | Primary | Primary |  |  |

ISM = Industrial, Scientific and Medical.

(1) Until 31 December 2006.

The Schedule to Terms, Provisions and Limitations Booklet BR68/F
(Amateur Radio (Foundation) Licence)

| 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- |
| Frequency Bands in MHz | Status of Allocations in the United Kingdom to the Amateur Service | Status of Allocations in the United Kingdom to the Amateur Satellite Service | Power | Mode of Transmission |
| 0.1357-0.1378 | Secondary. Available on the basis of non-interference to other services (inside or outside the United Kingdom) | (not allocated) | 1 W (0 dBW) erp | Morse Telephony RTTY Data Facsimile SSTV |
| 1.810-1.830 | Primary. Available on the basis of non-interference to other services (outside the United Kingdom) |  | Power fed to the Antenna 10 W (10 dBW) |  |
| 1.830-1.850 | #### Primary |  |  |  |
| 1.850-2.000 | Available on the basis of non-interference to other services (inside or outside the United Kingdom) |  |  |  |
| 3.500-3.800 | Primary. Shared with other services |  |  |  |
| 7.000-7.100 | #### Primary |  |  |  |
| 10.100-10.150 | #### Secondary |  |  |  |
| 14.000-14.250 | #### Primary |  |  |  |
| 14.250-14.350 |  |  |  |  |
| 18.068-18.168 |  |  |  |  |
| 21.000-21.450 |  |  |  |  |
| 24.890-24.990 |  |  |  |  |
| 50.00-51.00 | Primary. Available on the basis of non-interference to other services outside the United Kingdom |  |  |  |
| 51.00-52.00 | Secondary. Available on the basis of non-interference to other services inside or outside the United Kingdom |  |  |  |
| 70.00-70.50 |  |  |  |  |
| 144.0-146.0 | #### Primary |  |  |  |
| 430.0-431.0 | Secondary |  | 10 W (10 dBW) erp |  |
| 431.0-432.0 | Secondary. Not available for use within a 100 km radius of Charing Cross, London (51°30´30´´N, 00°07´24´´W) |  |  |  |
| 432.0-438.0 | #### Secondary |  | 10 W (10 dBW) |  |
| 438.0-440.0 | Secondary |  |  |  |